# AVR ATmega Bare-Metal Programming – Assignment Questions & Projects

Target MCU: ATmega328P / ATmega32 (or equivalent AVR ATmega series)
Programming Style: Bare-metal (Register-level, no Arduino libraries)
Language: Embedded C (avr-gcc)
Focus: Understanding hardware through datasheet-driven programming

## Assignment Set 1: GPIO – Input, Output & Alternate Functions

- Write a bare-metal program to configure PORTB pins PB0–PB3 as output and PB4–PB7 as input. Toggle output pins when corresponding input pins are pressed.

  - Concepts: DDRx, PORTx, PINx

- Configure one GPIO pin as open-drain equivalent using DDR and PORT registers. Interface an external pull-up resistor and verify behavior.

  - Concepts: GPIO electrical behavior

- Develop a program to generate a square wave of 1 kHz on an output pin using software delay only. Measure frequency using CRO/logic analyzer.

  - Concepts: GPIO toggling, instruction cycle timing

- Write a program to configure a pin for alternate function (Timer Output Compare – OC0/OC1) and generate a PWM signal.

  - Concepts: Pin multiplexing, alternate pin function

- Implement bitwise manipulation macros to set, clear, toggle, and read GPIO pins. Use them in a sample application.

  - Concepts: Bit masking, macros

## Assignment Set 2: Timers and Counters

- Configure Timer0 in Normal Mode to generate a delay of 10 ms using polling. Toggle an LED every delay.

  - Concepts: Prescaler, TCNT, overflow flag

- Use Timer1 in CTC mode to generate a precise 1-second delay. Do not use software loops.

  - Concepts: CTC, OCR register

- Develop a program to generate PWM using Timer0 Fast PWM mode with variable duty cycle controlled by switches.

  - Concepts: PWM modes, duty cycle control

- Write a program to measure external signal frequency using Timer1 as a counter.

  - Concepts: External clock source, counting

- Compare delay accuracy between software delay and hardware timer delay. Document results.

## Assignment Set 3: Interrupts & ISR Handling

- Configure Timer0 overflow interrupt to toggle an LED every 100 overflows.

- Concepts: ISR, global interrupt enable

- Write a program to blink LED using interrupt-driven timer while main loop performs a dummy task.

- Concepts: Interrupt vs polling

- Implement nested interrupt-safe code using volatile variables. Explain why volatile is required.

- Create a program where two different interrupts (Timer + External) update different variables.

- Concepts: Multiple ISRs

- Demonstrate the effect of disabling and enabling global interrupts inside critical sections.

## Assignment Set 4: External Interrupts & Peripheral Interfaces

- Configure INT0 for falling-edge triggering. Toggle an LED on each external interrupt.

- Concepts: EICRA, EIMSK

- Interface a push button using external interrupt and debounce it using software logic.

- Use Pin Change Interrupt (PCINT) to detect changes on multiple pins.

- Interface a relay module using GPIO and external interrupt-based safety shutdown.

- Design a program to count the number of external interrupt occurrences and display the count using LEDs.

## Assignment Set 5: ADC and DAC Interfaces

- Configure ADC to read potentiometer voltage using polling. Display result via LEDs (bar graph).

- Concepts: ADCMUX, ADCSRA

- Implement ADC with interrupt mode and store sampled values in an array.

- Change ADC reference voltage between AVCC and internal reference and compare readings.

- Generate a DAC output using PWM + RC filter to approximate analog voltage.

- Concepts: PWM as DAC

- Implement temperature sensing using LM35 sensor and ADC.

## Assignment Set 6: Display & Keypad Interfaces

- Interface single 7-segment display using GPIO. Display digits 0–9.

- Interface multiplexed 4-digit 7-segment display using timer interrupt for refreshing.

- Concepts: Multiplexing, ISR timing

- Interface 16x2 LCD in 4-bit mode without using libraries.

- Interface 4x4 matrix keypad and detect key presses using row-column scanning.

- Develop a menu-based system using keypad and LCD.

## Assignment Set 7: External Sensor Signal Handling

- Interface an LDR using ADC and control LED brightness.

- Interface an ultrasonic sensor using timer input capture.

- Concepts: Pulse width measurement

- Read digital sensor output using GPIO interrupt.

- Implement sensor data averaging to reduce noise.

- Create a threshold-based alert system using sensor input.

# Mini Projects (Bare-Metal Focus)

## Project 1: Breadboard-Based AVR System Bring-Up

Objective: Build and validate a complete ATmega system on breadboard.

Tasks:

- Power supply and decoupling

- Clock source (internal/external crystal)

- Reset circuitry

- GPIO LED blink

- External interrupt validation

Deliverables:

- Circuit diagram

- Register-level code

- Debug observations

## Project 2: Traffic Light Controller (FSM-Based)

Features:

- Timers for delays

- External interrupt for emergency override

- LED-based traffic signals

Concepts:

- Finite State Machine (FSM)

- Modular programming

- Interrupt-driven design

## Project 3: Digital Voltmeter using ADC

Features:

- ADC sampling

- LCD display

- Calibration using reference voltage

Concepts:

- ADC accuracy

- Data formatting

## Project 4: Smart Keypad-Controlled Load System

Features:

- Keypad password entry

- Relay control

- Timer-based lockout

Concepts:

- FSM

- Security logic

- Interrupts