## 1. Programs to Understand Various Clock Configurations (STM32)

- Write a program to configure the STM32 system clock using HSI as the clock source and blink an LED.
 Concepts to use: RCC, clock source selection, GPIO

- Configure the system clock to use HSE (external crystal) and verify the clock change using LED toggle delay.
 Concepts: RCC_CR, RCC_CFGR, HSE ready flag

- Configure the PLL to generate a higher system clock frequency from HSE and verify using SysTick delay.
 Concepts: PLL configuration, prescalers, SysTick

- Write a program to change AHB, APB1, and APB2 prescalers and observe the effect on timer speed.
 Concepts: Bus clocks, timer clock doubling

- Implement a program that switches the system clock dynamically at runtime (HSI → PLL).
 Concepts: Clock safety, clock switching sequence

## 2. Programs to Understand Timer Configurations and Event Handlers

- Configure a basic timer to generate a periodic interrupt every 1 second and toggle an LED.
 Concepts: Timer prescaler, auto-reload, ISR

- Write a program using general-purpose timer to generate a precise delay without using HAL_Delay().
 Concepts: Timer polling, counter overflow

- Configure a timer in PWM mode to control LED brightness using duty cycle variation.
 Concepts: PWM mode, CCR register

- Use a timer in output compare mode to generate a square wave on a GPIO pin.
 Concepts: Output compare, alternate function GPIO

- Implement a timer-based event handler that triggers an action every N timer events.
 Concepts: Timer interrupts, event counting

## 3. Programs to Understand Interrupts and Interrupt Service Routines (ISR)

- Write a program where a timer interrupt increments a counter and displays it using LED patterns.
 Concepts: ISR, global variables, volatile

- Demonstrate the difference between polling and interrupt-based LED blinking using a timer.
 Concepts: Blocking vs non-blocking execution

- Configure two interrupts with different NVIC priorities and observe preemption behavior.
 Concepts: NVIC priority grouping

- Write an ISR that performs minimal processing and defers work to the main loop using flags.
 Concepts: ISR best practices

- Implement an interrupt that triggers after a fixed number of timer overflows.
 Concepts: Interrupt counting, state management

## 4. Programs to Understand External Interrupts and External Peripheral Interface

- Configure a GPIO pin as external interrupt (EXTI) and toggle an LED on button press.
 Concepts: EXTI lines, SYSCFG, NVIC

- Write a program to detect rising and falling edges on an external interrupt pin.
 Concepts: Edge detection, EXTI trigger selection

- Interface a push button with debounce handling using external interrupts.
 Concepts: Software debounce, EXTI masking

- Configure an external interrupt to wake the microcontroller from low-power mode.
 Concepts: Sleep/Stop mode, wake-up interrupt

- Interface an external peripheral (e.g., IR sensor or limit switch) using EXTI and control an output device.
 Concepts: Peripheral interface, interrupt-driven control

## 5. Programs to Understand Handling of External Signals

- Use a timer in input capture mode to measure the frequency of an external signal.
 Concepts: Input capture, signal timing

- Measure the pulse width of an external signal using timer input capture.
 Concepts: Rising/falling capture, CCR registers

- Read an analog external signal using ADC with interrupt and process the data.
 Concepts: ADC, ISR, sampling

- Configure ADC with DMA to continuously sample an external sensor signal.
 Concepts: DMA, circular buffer