## Section 1: Basic Resistive Networks (R, Series, Parallel) – 15 Questions

- Compute total resistance for 3 resistors in series.
C Implementation: Use variables or an array: float R[3]; float R_total = 0; for(i=0;i<3;i++) R_total += R[i];

- Compute total resistance for 3 resistors in parallel.
C Implementation: float R_total = 1/(1/R[0] + 1/R[1] + 1/R[2]);

- Find voltage across a resistor in a series network.
C Implementation: V_resistor = I_total * R[i];

- Find current through a resistor in a parallel network.
C Implementation: I_resistor = V_source / R[i];

- Calculate power dissipated in a resistor.
C Implementation: P = V*V/R;

- Compute total resistance of a combination of series and parallel resistors.
C Implementation: Use functions: float series(int n, float arr[]); float parallel(int n, float arr[]);

- Compute current through each resistor in a complex network.
C Implementation: Use loops and arrays: for(i=0;i<n;i++) I[i] = V/R[i];

- Simulate a voltage divider and calculate output voltage.
C Implementation: V_out = V_in * R2/(R1+R2);

- Simulate a current divider and calculate branch currents.
C Implementation: I_branch = I_total * R_total/R_branch;

- Compute equivalent resistance of a cube of resistors.
C Implementation: Represent resistors as 3D array or adjacency matrix; use formula for symmetry.

- Find resistance between two nodes in a lattice.
C Implementation: Use arrays to represent connections; solve using series-parallel rules or iterative method.

- Function to calculate series resistance for N resistors.
C Implementation: float series(int n, float R[]) { float sum=0; for(i=0;i<n;i++) sum+=R[i]; return sum; }

- Function to calculate parallel resistance for N resistors.
C Implementation: float parallel(int n, float R[]) { float sum=0; for(i=0;i<n;i++) sum+=1/R[i]; return 1/sum; }

- Verify Ohm’s law for network with random voltage and resistors.
C Implementation: I = V/R; printf("Current: %f\n", I);

- Compute average power dissipated in resistive network.
C Implementation: for(i=0;i<n;i++) P_total += I[i]*I[i]*R[i]; P_avg = P_total/n;

## Section 2: Kirchhoff’s Laws (KVL & KCL) – 15 Questions

- Apply KCL at a node; find unknown current.
C Implementation: I_unknown = I1 + I2 - I3;

- Apply KVL around a loop; find unknown voltage.
C Implementation: V_unknown = V_source - sum(V_other);

- Solve two-loop circuit using KVL.
C Implementation: Represent voltages as array; solve linear equations using loops or matrices.

- Solve three-node circuit using KCL.
C Implementation: Represent currents as array; solve node voltage equations.

- Simulate current distribution in 4-node network.
C Implementation: Use arrays and for-loops to compute branch currents.

- Function to automate KCL summation for N nodes.

float kcl_sum(int n, float I[]) {

float sum=0; for(i=0;i<n;i++) sum+=I[i]; return sum;

}

Function to automate KVL summation for M loops.

float kvl_sum(int m, float V[]) {

float sum=0; for(i=0;i<m;i++) sum+=V[i]; return sum;

}

- Verify KCL for parallel resistor network.
C Implementation: sum(I_branches) == I_total;

- Verify KVL for series-parallel network.
C Implementation: sum(V_resistors) == V_source;

- Compute unknown voltage in multi-loop circuit.
C Implementation: Solve using arrays and loops.

- Compute unknown current in multi-node circuit.
C Implementation: Solve linear equations in C using arrays or structs.

- Simulate branch currents in series-parallel network.
C Implementation: Use I = V/R for each branch.

- Compute node voltages using node voltage method.
C Implementation: Set up N×N matrix; solve linear system using Gauss elimination.

- Compute loop currents using mesh current method.
C Implementation: Use loop matrix; solve equations.

- General solver for N-node resistive network using KCL.
C Implementation: Use float node_voltage[N]; with iterative solver.

## Section 3: Thevenin & Norton Theorems – 10 Questions

- Compute Thevenin voltage/resistance of 2-resistor network.
C Implementation: V_th = V_open; R_th = R1||R2;

- Compute Norton current/resistance of 2-resistor network.
C Implementation: I_n = V_th/R_th;

- Function to convert Thevenin to Norton.

void thev_to_norton(float Vth, float Rth, float *In, float *Rn){

*In = Vth / Rth; *Rn = Rth;

}

- Simulate load current using Thevenin equivalent.
C Implementation: I_load = V_th/(R_th + R_load);

- Simulate load voltage using Norton equivalent.
C Implementation: V_load = I_n * R_load * R_parallel(Rn, R_load);

- Maximum power transfer to load resistor.
C Implementation: R_load = R_th; P_max = (V_th*V_th)/(4*R_th);

- Compute power dissipated in load using Thevenin.
C Implementation: P = I_load*I_load*R_load;

- Thevenin equivalent for network with 3 resistors and 1 voltage source.
C Implementation: Solve using series-parallel reduction in C.

- Simulate Norton current division in parallel network.
C Implementation: I_branch = I_n * R_total / R_branch;

- Verify Thevenin theorem by comparing load voltage/current with original network.
C Implementation: Compare V_load or I_load.

## Section 6: Transient Analysis (RC, RL, RLC circuits) – 10 Questions

- Compute voltage across a capacitor in RC charging circuit.
C Implementation: Use Vc(t) = V*(1-exp(-t/(R*C))) in a loop over time steps.

- Compute current in RL circuit during switch-on.
C Implementation: I(t) = (V/R)*(1-exp(-R*t/L)); loop over time steps.

- Simulate RC charging curve using time steps.
C Implementation: for(t=0; t<Tmax; t+=dt) Vc = V*(1-exp(-t/(R*C)));

- Simulate RC discharging curve.
C Implementation: Vc = V0*exp(-t/(R*C)); in loop.

- Simulate RL circuit response to a step input.
C Implementation: I = (V/R)*(1-exp(-R*t/L)); loop over t.

- Compute time constant for RC and RL circuits.
C Implementation: tau_RC = R*C; tau_RL = L/R;

- Simulate series RLC circuit response to step input.
C Implementation: Use iterative loop to calculate Vc or IL with differential equations.

- Compute natural frequency of series RLC: f0 = 1/(2*pi*sqrt(L*C))
C Implementation: Use math.h functions sqrt() and M_PI.

- Compute damped response of RLC series circuit.
C Implementation: alpha = R/(2*L); omega0 = 1/sqrt(L*C); then V = V0*exp(-alpha*t)*cos(sqrt(omega0^2-alpha^2)*t);

- Plot transient response for RC, RL, RLC circuits.
C Implementation: Store results in array float V_array[Tmax/dt]; then print to file for plotting.

## Section 7: AC Steady-State Analysis (Phasors, Impedance) – 10 Questions

- Compute impedance of series RLC at given frequency.
C Implementation: Z = sqrt(R*R + pow(omega*L-1/(omega*C),2));

- Compute current in series AC circuit using phasors.
C Implementation: I = V_source / Z;

- Compute voltage across R, L, C in series AC.
C Implementation: VR = I*R; VL = I*omega*L; VC = I/(omega*C);

- Compute power factor in series AC circuit.
C Implementation: pf = cos(atan((omega*L-1/(omega*C))/R));

- Function to compute phasor voltage and current.

void phasor(float V, float R, float L, float C, float f, float *I, float *VR, float *VL, float *VC){

float omega = 2*M_PI*f;

*I = V/sqrt(R*R + pow(omega*L - 1/(omega*C),2));

*VR = (*I)*R;

*VL = (*I)*omega*L;

*VC = (*I)/(omega*C);

}

- Simulate parallel RLC branch currents.
C Implementation: IR = V/R; IL = V/(omega*L); IC = V*omega*C; I_total = sqrt(pow(IR,2)+pow(IL-IC,2));

- Compute total impedance of parallel RLC.
C Implementation: 1/Z_total = 1/R + 1/(j*omega*L) + j*omega*C; store real and imaginary separately.

- Compute apparent, active, reactive power in AC circuit.
C Implementation: S = V*I; P = S*pf; Q = S*sin(acos(pf));

- Verify phasor sum of voltages equals source voltage.
C Implementation: V_total = sqrt(pow(VR,2) + pow(VL-VC,2));

- Simulate series AC circuit for multiple frequencies.
C Implementation: Loop over frequency array, compute Z, I, VR, VL, VC.

## Section 8: Frequency Response & Filters – 10 Questions

- Voltage across capacitor in RC low-pass filter for range of frequencies.
C Implementation: Vc = V_in * 1/sqrt(1+pow(f/fc,2));

- Voltage across resistor in RC high-pass filter.
C Implementation: VR = V_in * pow(f/fc,2)/sqrt(1+pow(f/fc,2));

- Compute cutoff frequency for RC filter: fc = 1/(2*pi*R*C)

- Simulate band-pass filter output.
C Implementation: Use f_center = 1/(2*pi*sqrt(L*C)); and V_out = V_in * function(f)

- Compute resonant frequency for series RLC: f0 = 1/(2*pi*sqrt(L*C));

- Plot magnitude response of RC, RL, RLC circuits.
C Implementation: Store V_array[i] for frequency sweep; output to CSV.

- Compute phase response: phi = atan((omega*L-1/(omega*C))/R);

- Digital filter simulation using difference equation:
C Implementation: y[n] = a*x[n] + b*x[n-1];

- Simulate attenuation in RC network: V_out/V_in = 1/sqrt(1+(f/fc)^2);

- Simulate RLC filter response for multiple frequencies.
C Implementation: Loop over f_array[]; compute Z and I for each.

## Section 9: Power Calculations – 5 Questions

- Compute average power in resistive AC circuit: P = I_rms*I_rms*R;

- Compute reactive power: Q = V_rms*I_rms*sin(phi);

- Compute apparent power: S = V_rms*I_rms;

- Compute power factor correction needed.
C Implementation: pf_new = desired_pf; Qc = P*(tan(phi_old)-tan(phi_new));

- Compute total power in multi-branch AC network.
C Implementation: Sum P, Q, S for each branch using arrays.

## Section 10: Transmission Line & Miscellaneous – 10 Questions

- Compute R, L, C, G per unit length.
C Implementation: Use input arrays for physical parameters; formulas: R = resistivity*l/A; L = mu*l; C = epsilon*A/l; G = sigma*A/l;

- Compute characteristic impedance: Z0 = sqrt((R+j*omega*L)/(G+j*omega*C));

- Compute propagation constant: gamma = sqrt((R+j*omega*L)*(G+j*omega*C));

- Voltage along lossless line: V(z) = V0+*exp(-j*beta*z) + V0-*exp(j*beta*z);

- Current along line: I(z) = (V0+/Z0)*exp(-j*beta*z) - (V0-/Z0)*exp(j*beta*z);

- Simulate reflection coefficient: Gamma = (ZL-Z0)/(ZL+Z0);

- Simulate standing wave ratio (SWR): SWR = (1+|Gamma|)/(1-|Gamma|);

- Compute input impedance at distance d: Z_in = Z0*(ZL+j*Z0*tan(beta*d))/(Z0+j*ZL*tan(beta*d));

- Simulate transient on transmission line with RC load.
C Implementation: Loop over discrete time steps: V[n+1] = V[n] + dt*(I/C);

- Simulate voltage and current distribution for multiple line sections.
C Implementation: Use arrays for each section; apply transmission line equations iteratively.