Simple Hello Kernel Module program for Linux along with the commands to compile, insert, verify, and remove it.

1. hello_kernel.c

#include <linux/init.h>
#include <linux/module.h>
#include <linux/kernel.h>

MODULE_LICENSE("GPL");
MODULE_AUTHOR("Mohan");
MODULE_DESCRIPTION("Simple Hello Kernel Module");
MODULE_VERSION("1.0");

/* Function called when module is loaded */
static int __init hello_init(void)
{
    printk(KERN_INFO "Hello: Kernel Module Loaded\n");
    return 0;
}

/* Function called when module is removed */
static void __exit hello_exit(void)
{
    printk(KERN_INFO "Hello: Kernel Module Removed\n");
}

module_init(hello_init);
module_exit(hello_exit);

2. Makefile

Create a file named Makefile

obj-m += hello_kernel.o

KDIR = /lib/modules/$(shell uname -r)/build
PWD  = $(shell pwd)

all:
	make -C $(KDIR) M=$(PWD) modules

clean:
	make -C $(KDIR) M=$(PWD) clean

3. Compile the Module

Open terminal in the same directory and run:

make

After compilation, you should get:

hello_kernel.ko

4. Insert the Module

sudo insmod hello_kernel.ko

5. Check Whether Module is Loaded

Method 1 — Using lsmod

lsmod | grep hello_kernel

Example output:

hello_kernel     16384   0

Method 2 — Check Kernel Messages

dmesg | tail

Expected output:

Hello: Kernel Module Loaded

6. Remove the Module

sudo rmmod hello_kernel

7. Verify Removal

dmesg | tail

Expected output:

Hello: Kernel Module Removed

Or:

lsmod | grep hello_kernel

No output means module removed successfully.

8. Important Notes

If you get:

module verification failed: signature and/or required key missing

it is usually normal on systems with Secure Boot disabled or unsigned modules. The module still loads.

9. Install Required Packages (Ubuntu/Debian)

If compilation fails, install kernel headers:

sudo apt update
sudo apt install build-essential linux-headers-$(uname -r)

Folder Structure

hello_module/
│
├── hello_kernel.c
└── Makefile

Explanation of the Kernel Module Program

#include <linux/init.h>
#include <linux/module.h>
#include <linux/kernel.h>

These are Linux kernel header files.

- linux/init.h

  - Contains macros like __init and __exit

- linux/module.h

  - Required for all kernel modules

  - Provides module-related functions/macros

- linux/kernel.h

  - Contains kernel logging functions like printk()

Module Information

MODULE_LICENSE("GPL");
MODULE_AUTHOR("Mohan");
MODULE_DESCRIPTION("Simple Hello Kernel Module");
MODULE_VERSION("1.0");

These macros provide metadata about the module.

| Macro | Purpose |
| --- | --- |
| MODULE_LICENSE | Specifies module license |
| MODULE_AUTHOR | Author name |
| MODULE_DESCRIPTION | Description |
| MODULE_VERSION | Module version |

Module Initialization Function

static int __init hello_init(void)
{
    printk(KERN_INFO "Hello: Kernel Module Loaded\n");
    return 0;
}

This function runs when the module is inserted using:

sudo insmod hello_kernel.ko

Breakdown

static

Limits the function visibility to this source file only.

Without static, the function becomes globally visible in the kernel symbol table.

Kernel modules usually keep helper functions private unless they must be shared.

int

The function returns an integer.

- 0 → success

- non-zero → loading failed

Example:

return -1;

would prevent module loading.

__init

Special kernel macro.

It tells the kernel:

"This function is needed only during module initialization."

After loading completes successfully, the memory occupied by this function can be freed.

This saves kernel memory.

printk()

Kernel-space version of printf().

printk(KERN_INFO "Hello: Kernel Module Loaded\n");

- KERN_INFO → log level

- Message appears in kernel logs (dmesg)

Module Exit Function

static void __exit hello_exit(void)
{
    printk(KERN_INFO "Hello: Kernel Module Removed\n");
}

Runs when module is removed:

sudo rmmod hello_kernel

Why void?

The cleanup function does not return anything.

Why __exit?

Marks the function as cleanup code.

For built-in kernel drivers, this code may be discarded if unloading is impossible.

Module Registration Macros

module_init(hello_init);
module_exit(hello_exit);

These macros tell the kernel:

| Macro | Meaning |
| --- | --- |
| module_init() | Entry function |
| module_exit() | Exit function |

Why static is Important

Example Without static

int hello_init(void)
{
    return 0;
}

This function becomes globally visible.

Problems:

- Name conflicts with other kernel modules

- Unnecessary symbol exposure

- Larger kernel symbol table

Example With static

static int hello_init(void)
{
    return 0;
}

Now:

- Function accessible only inside this .c file

- Better encapsulation

- Safer kernel programming

- Reduces namespace pollution

Real-Time Analogy

Think of static like a private method in OOP.

| Without static | With static |
| --- | --- |
| Publicly accessible | Private to file |
| Can conflict | Safe/internal |
| Exported symbol | Hidden symbol |

Execution Flow

insmod hello_kernel.ko
        ↓
hello_init() executes
        ↓
Message printed to kernel log
        ↓
Module stays loaded
        ↓
rmmod hello_kernel
        ↓
hello_exit() executes
        ↓
Cleanup message printed

Important Difference: User Space vs Kernel Space

| User Space | Kernel Space |
| --- | --- |
| printf() | printk() |
| libc available | No libc |
| Safer crashes | Kernel crash possible |
| Normal memory | Kernel memory |

Output Checking

dmesg | tail

Example:

Hello: Kernel Module Loaded
Hello: Kernel Module Removed