Project Statement: I2C Communication Simulator with Operating System Concepts in C

Objective

Design and implement a software-based I2C communication simulator in C that integrates core Operating System concepts such as processes, threads, synchronization, inter-process communication (IPC), scheduling, signals, shared memory, message queues, semaphores, and timers. The project shall simulate an I2C bus environment consisting of multiple masters and slave devices (Memory, FIFO, and LIFO) executing as independent OS-managed entities.

System Architecture

+---------------------------------------------------+
|                 I2C BUS PROCESS                   |
|                                                   |
|    Shared Memory + Mutex + Semaphore              |
+---------------------------------------------------+
          ^              ^               ^
          |              |               |
          |              |               |
+---------+---+  +-------+-----+  +------+------+
| MASTER 1    |  | MASTER 2    |  | BUS MONITOR |
|  Process    |  |  Process    |  |   Thread    |
+-------------+  +-------------+  +-------------+

          |
          |
-----------------------------------------------------
|                    I2C BUS                         |
-----------------------------------------------------
      |               |                 |
      |               |                 |
+-----+-----+   +-----+-----+    +------+------+
| Memory     |   | FIFO      |    | LIFO       |
| Process    |   | Process   |    | Process    |
+------------+   +-----------+    +------------+

OS Concepts to be Implemented

1. Process Management

- Implement each I2C device as an independent process.

- Create processes using fork().

- Parent process acts as system controller.

- Child processes represent:

  - I2C Master 1

  - I2C Master 2

  - Memory Slave

  - FIFO Slave

  - LIFO Slave

Learning Outcome

- Process creation

- Parent-child relationships

- Process lifecycle management

2. Multithreading

Each process shall internally create multiple threads.

Example

Master Process

- Command Generation Thread

- Data Transfer Thread

- Error Monitoring Thread

Slave Process

- Request Handler Thread

- Data Processing Thread

- Status Monitoring Thread

Learning Outcome

- POSIX Threads (pthreads)

- Concurrent execution

- Thread synchronization

3. Shared Memory IPC

Implement the I2C Bus as a shared memory segment.

Shared Data

struct I2C_Bus
{
    uint8_t address;
    uint8_t data;
    uint8_t rw;

    uint8_t ack;

    uint8_t bus_busy;
};

Operations

- Masters write transactions to bus.

- Slaves monitor bus and respond.

Learning Outcome

- shm_open()

- mmap()

- Shared resource management

4. Mutex Synchronization

Protect bus access using mutexes.

Scenario

Two masters attempt bus access simultaneously.

pthread_mutex_lock(&bus_lock);

/* Access Bus */

pthread_mutex_unlock(&bus_lock);

Learning Outcome

- Critical sections

- Race condition prevention

5. Semaphores

Control access to FIFO and LIFO buffers.

FIFO Example

sem_wait(&fifo_empty);

enqueue();

sem_post(&fifo_full);

Learning Outcome

- Producer-Consumer Problem

- Resource synchronization

6. Message Queues

Implement command transfer between processes using POSIX Message Queues.

Commands

WRITE MEMORY
READ MEMORY

FIFO PUSH
FIFO POP

LIFO PUSH
LIFO POP

Learning Outcome

- mq_open()

- mq_send()

- mq_receive()

7. Signals

Implement asynchronous event notifications.

Examples

FIFO Full

SIGUSR1

FIFO Empty

SIGUSR2

Bus Error

SIGINT

Learning Outcome

- signal()

- sigaction()

8. Timers

Simulate I2C clock timing using POSIX timers.

Features

- Bus timeout detection

- Clock generation

- Transaction timeout monitoring

Learning Outcome

- timer_create()

- timer_settime()

9. Scheduling Concepts

Simulate priority-based scheduling.

Priorities

| Process | Priority |
| --- | --- |
| Bus Manager | High |
| Master | High |
| Memory Slave | Medium |
| FIFO Slave | Medium |
| LIFO Slave | Medium |
| Logger | Low |

Learning Outcome

- sched_setscheduler()

- Real-time scheduling concepts

10. Producer-Consumer Problem

FIFO Slave shall implement classical Producer-Consumer architecture.

Producer

Master writes data.

Consumer

Master reads data.

Synchronization

- Mutex

- Semaphores

Learning Outcome

- Bounded Buffer Problem

11. Reader-Writer Problem

Memory Slave shall support:

Readers

Multiple masters reading memory.

Writers

Single master writing memory.

Synchronization

pthread_rwlock_t

Learning Outcome

- Reader-Writer Lock

12. Event Logging Thread

Create a dedicated logging thread.

Logs

Timestamp
Process ID
Thread ID
Slave Address
Operation
ACK/NACK
Status

Learning Outcome

- Thread-safe logging

- System monitoring

Functional Modules

Module 1: I2C Bus Manager

Responsibilities:

- Bus arbitration

- Address decoding

- Clock simulation

- Shared memory management

Module 2: Master Controller

Responsibilities:

- Transaction generation

- Multi-byte transfers

- Error recovery

Module 3: Memory Slave

Responsibilities:

- Memory access

- Reader-Writer synchronization

Module 4: FIFO Slave

Responsibilities:

- Queue management

- Producer-consumer implementation

Module 5: LIFO Slave

Responsibilities:

- Stack management

- Synchronization

Module 6: Monitoring & Diagnostics

Responsibilities:

- Event logging

- Performance statistics

- Error reporting

Advanced Features

- Multi-master arbitration simulation

- Dynamic slave registration

- Slave-to-master role switching

- Deadlock detection

- Watchdog timer implementation

- CPU utilization monitoring

- Transaction throughput analysis

- Real-time scheduling policies

Expected Deliverables

- Complete modular C project.

- Process-based I2C simulation framework.

- Threaded master and slave implementations.

- Shared memory bus simulator.

- Message queue-based communication.

- Semaphore and mutex synchronization.

- Signal handling framework.

- POSIX timer implementation.

- Event logging subsystem.

- Makefile and build scripts.

- System design documentation.

- Performance analysis report.

OS Concepts Covered

| OS Concept | Usage in Project |
| --- | --- |
| Processes | Master and Slave Devices |
| Threads | Concurrent Transaction Handling |
| Shared Memory | I2C Bus Simulation |
| Mutexes | Bus Access Protection |
| Semaphores | FIFO/LIFO Synchronization |
| Message Queues | Command Transfer |
| Signals | Event Notifications |
| Timers | Clock & Timeout Simulation |
| Scheduling | Priority-Based Execution |
| Reader-Writer Locks | Memory Slave Access |
| Producer-Consumer | FIFO Implementation |
| Deadlock Avoidance | Bus Arbitration |
| Logging | System Monitoring |

This version transforms the project from a simple protocol simulator into a comprehensive Operating Systems Laboratory + Embedded Systems + IPC + Synchronization project suitable for advanced Linux/QNX OS programming coursework.